/*---------------------------------------------------------------------------

|

|    Documentation for global routines/structures.

|

| Jose' Monteiro, MIT, Mar/94            jcm@rle-vlsi.mit.edu

+--------------------------------------------------------------------------*/

Description:

The power package for SIS.

Summary:

power_estimate();

power_main_driver();

power_sample_estimate();

power_simulation_estimate();

power_free_info();

power_print();

SWITCH_PROB();

CAP_FACTOR();

int

power_estimate(network, type, delay, total_power)

network_t *network;

int type;             /* {COMBINATIONAL; SEQUENTIAL; PIPELINE; DYNAMIC} */

int delay;            /* {0 - zerod delay; 1 - unit delay; 2 - general delay} */

double *total_power;  /* Returned value */

Calls power_main_driver with default values:

psProbOption = APPROXIMATION_PS;

info_file = NIL(FILE);

sop_or_fact = FACTORED_FORM;

input_sizing = DEFAULT_MAX_INPUT_SIZING (4);

power_setSize = 1;

power_delta = DEFAULT_PS_MAX_ALLOWED_ERROR (0.01);

power_verbose = 0;

power_cap_in = CAP_IN_LATCH (4);

power_cap_out = CAP_OUT_LATCH (20);

int

power_main_driver(network, type, delay, psProbOption, info_file,

sop_or_fact, input_sizing, total_power)

network_t *network;

int type;             /* {COMBINATIONAL; SEQUENTIAL; PIPELINE; DYNAMIC} */

int delay;            /* {0 - zerod delay; 1 - unit delay; 2 - general delay} */

int psProbOption;     /* {APPROXIMATION_PS; EXACT_PS; UNIFORM_PS} */

FILE *info_file;      /* File with primary input probabilities */

int sop_or_fact;      /* {FACTORED_FORM; SUM_OF_PRODUCTS} */

int input_sizing;     /* Maximum number of inputs of node for sizing */

double *total_power;  /* Returned value */

NOTE: Besides these arguments, the calling routine must set global variables:

power_setSize, power_delta, power_verbose, power_cap_[in/out].

Estimates the capacitive load and expected number of transitions of each

node in the network. These values are stored in a global table and can

be retrieved using the macros SWITCH_PROB and CAP_FACTOR. The estimated

total power dissipated by the network is returned in "total_power". This

value is computed by adding SWITCH_PROB * CAP_FACTOR * CAPACITANCE * 250.0

over all the nodes (250.0 = 0.5 * 5V^2 * 20MHz and CAPACITANCE = 0.01).

The routine returns 0 if all went well, 1 if something went wrong.

int

power_sample_estimate(network, type, delay, num_samples, total_power)

network_t *network;

int type;             /* {COMBINATIONAL; SEQUENTIAL; PIPELINE; DYNAMIC} */

int delay;            /* {0 - zerod delay; 1 - unit delay; 2 - general delay} */

int num_samples;      /* Number of sets of 32 input vectors to simulate */

double *total_power;  /* Returned value */

Calls power_simulation_estimate with default values:

psProbOption = APPROXIMATION_PS;

info_file = NIL(FILE);

sop_or_fact = FACTORED_FORM;

input_sizing = DEFAULT_MAX_INPUT_SIZING (4);

power_setSize = 1;

power_delta = DEFAULT_PS_MAX_ALLOWED_ERROR (0.01);

power_verbose = 0;

power_cap_in = CAP_IN_LATCH (4);

power_cap_out = CAP_OUT_LATCH (20);

int

power_simulation_estimate(network, type, delay, psProbOption, num_samples,

sample_gap, info_file, sop_or_fact, input_sizing,

total_power)

network_t *network;

int type;             /* {COMBINATIONAL; SEQUENTIAL; PIPELINE; DYNAMIC} */

int delay;            /* {0 - zerod delay; 1 - unit delay; 2 - general delay} */

int psProbOption;     /* {APPROXIMATION_PS; EXACT_PS; UNIFORM_PS} */

int num_samples;      /* Number of sets of 32 input vectors to simulate */

int sample_gap;       /* Sample interval to print current power estimate */

FILE *info_file;      /* File with primary input probabilities */

int sop_or_fact;      /* {FACTORED_FORM; SUM_OF_PRODUCTS} */

int input_sizing;     /* Maximum number of inputs of node for sizing */

double *total_power;  /* Returned value */

Same as power_estimate, but uses logic simulation instead of calculating

signal probabilities through BDD's (can be less accurate and slower,

but can be used with larger circuits for which BDD's are too large).

int

power_free_info(network)

network_t *network;

Frees power information stored in global table 'power_info_table'.

Always returns 0.

int

power_print(network)

network_t *network;

Goes through all nodes of the network and prints their switching

probability and capacitance.

double

SWITCH_PROB(node)

node_t *node;

Returns the expected number of transitions this node makes

per clock cycle.

int

CAP_FACTOR(node);

node_t *node;

Returns the capacitive load the node is driving.